API 653 Overhaul Procedures for a Varec 2500 ATG

The following procedure references the 2500 Automatic Tank Gauge Installation, Operations and Service Manual (Document code: IOM001GVAE1108. Download the PDF file online at http://www.tankgauging.com/_docs/manuals/IOM001_2500c.pdf

- Determine your installation/tank type - Refer to section 2.4.

- The Overhaul/ Refurbishing kit (part numbers 13-08770 English and 13-08771 Metric) provides recommended spare parts for the overhaul procedure.  Identify any additional replacement parts required - Refer to section 8.4.1

- Install/replace the float, tape/cable and guidewires - Refer to section 3

Note: Use the old tape to pull in the new tape.

- Service the 2500 ATG gaugehead components - Refer to section 6 for gaugehead disassembly and re-assembly

- The servicing technician should complete a 2500 ATG Detailed Tank Inspection Report (FRM056) for each tank to record the service work completed.

- The 2500 ATG Overhaul Checklist (FRM185) can be used by an inspector to record the operation of the gauge system after service work is complete.

Note! Varec offers factory trained service personnel and provides a two year warranty on the repair – Contact the Varec Service Department for more details.